# Error Detection & Correction Codes

## Introduction

Error detection and correction codes ensure data is transmitted or stored accurately by detecting and correcting bit errors.

## Types of Errors

1. Single-bit error: only one bit changes.
2. Burst error: two or more consecutive bits change.

## Parity Checking

A parity bit is added to detect transmission errors.

## Even Parity

The total number of 1s including the parity bit is even.

## Odd Parity

The total number of 1s including the parity bit is odd.

## Hamming Code

Hamming code detects and corrects single-bit errors using parity bits placed at positions 1, 2, 4, 8, ...
Formula: 2^p >= m + p + 1
Steps:
• Calculate parity bits
• Insert parity bits
• Compute parity
• Recheck at receiver
• Correct erroneous bit using syndrome.

## Comparison

Parity checking detects errors only. Hamming code detects and corrects single-bit errors.